- A/T FLUID REPLACEMENT

|  | caution; Before replacing the transmission with a new one, overhauling the existing transmission, or connecting the cooler pipe to the transmission, the A/T fluid cooler line should always be flushed out.. |
| --- | --- |

|  | In you have an A/T fluid changer, use the A/T fluid changer to flush the A/T fluid. If you do not have an A/T fluid changer, follow the procedure given below. |
| --- | --- |

|  | 1.Remove the hose shown in the illustration which allows the A/T fluid to flow from the A/T fluid cooler to the transmission. 2. caution; The engine should be stopped within one minute of it being started. If the A/T fluid has all been discharged before this, stop the engine at that point.. Start the engine and discharge the A/T fluid. Driving conditions: N range, idling Discharge amount: Approx. 3.5 L |
| --- | --- |

|  | 3.Remove the drain plug at the bottom of the transmission case to drain out the remaining A/T fluid. Discharge amount: Approx. 2.0 L 4.Install the drain plug with a gasket in between, and tighten it to the specified torque. Tightening torque: 32 ± 2 N·m 5. caution; Stop pouring in the A/T fluid once 5.5 L has been poured in.. Pour the new A/T fluid in through the oil filler tube. Amount to add: Approx. 5.5 L 6.Repeat the operation in step 2. 7.Pour in new A/T fluid through the oil filler tube. Amount to add: Approx. 3.5 L note; Carry out steps 2 and 7 so that at least 8.0 L has been discharged from the cooler hose. After this, discharge a small quantity of A/T fluid and check for contamination. If the A/T fluid is contaminated, repeat steps 6 and 7.. |
| --- | --- |

|  | 8.Connect the hose which was disconnected in step 1, and then securely re-insert the oil level gauge. 9.Start the engine, and let it run at idle for 1 - 2 minutes. 10.Move the selector lever to all positions once, and then return it to the N position. |
| --- | --- |

|  | 11.Check that the A/T fluid level on the oil level gauge is at the COLD mark. if it is not up to this mark, add more A/T fluid. |
| --- | --- |

|  | 12.Drive the vehicle until the A/T fluid temperature reaches the normal temperature (70 - 80°C), and then re check the A/T fluid level. note; The COLD mark is for reference only; the HOT marks should be used as the standard for judgment.. note; A/T fluid temperature using M.U.T.-III.. note; Check the fluid level referring to the characteristics chart shown if it takes some time until reaching the normal operation temperature of A/T fluid (70 - 80°C.). 13.When A/T fluid is under the specified level, top up A/T fluid. When A/T fluid is over the specified level, drain the excessive A/T fluid from the drain plug to adjust A/T fluid level to the specified level. 14.Securely insert the oil level gauge into the oil filler tube. |
| --- | --- |