| ROAD SPRING DATA FOR ALL LAND ROVER VEHICLES |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Part | Load | Rate | Rate |  | Free | Free | No. of | Application |
| Number | (Lbs) | (Lbs/inch) | (Kgs/m) | Colour code | Lgth mm | Lgth ins | Coils | (Right Hand Drive Vehicles) |
| 572315 | 716 | 133 | 2375.1 | BLUE | 391.16 | 15.4 | 7.18 | Range Rover and Disco V8 std front |
| ANR1975 | 831.12 | 149.89 | 2876.61 | BLUE - RED - PINK |  |  |  | Discovery TDi RH front Vin KA to MA |
| ANR1976 | 922.62 | 149.88 | 2678.43 | BLUE - YELLOW - PINK |  |  |  | Discovery TDi LH front Vin KA to MA |
| ANR1977 | 753.11 | 207.84 | 3711.43 | RED - GREY |  |  |  | Disco rear Vin KA038878 to LA066328 - obsolete use ANR3058 |
| ANR3058 | 753.11 | 207.84 | 3711.43 |  |  |  |  | Disco rear Vin LA066329 to LA081991 |
| ANR3477 | 754.91 | 207.84 | 3711.43 | BROWN - GREEN |  |  |  | Disco rear Vin MA081991 to MA119533 |
| ANR3519 | 1069.87 | 149.03 | 2661.25 | RED - BROWN |  |  |  | Range Rover LHD diesel only |
| ANR3520 | 852.92 | 150.12 | 2680.71 | YELLOW - BROWN |  |  |  | Range Rover Classic rear from MA647645 |
| ANR3539 | 1007.14 | 182.72 | 3262.88 | WHITE |  |  |  | Military use only - No specifications available |
| ANR3540 | 2646 | 451.09 | 8055.18 | GREEN - PINK |  |  |  | Military use only - No specifications available |
| ANR3650 | 522.88 | 165.02 | 2946.79 | RED - ORANGE |  |  |  | Military use only - No specifications available |
| ANR4350 | 831.12 | 149.89 | 2676.61 | 2 BROWN |  |  |  | Disco front LH Vin MA115461 on |
| ANR4351 | 822.62 | 149.89 | 2676.61 | 2 GREY |  |  |  | Disco front RH Vin MA116461 on |
| ANR4352 | 754.91 | 165.88 | 2962.14 | 2 ORANGE |  |  |  | Disco rear- obsolete use 2 x ANR3477 |
| ANR4491 | 988.88 | 285.5 | 5098.21 | BROWN - GREY |  |  |  | Military use only - No specifications available |
| NRC2119 | 863 | 150 | 2676.7 | GREEN | 409.7 | 16.13 | 7.63 | Disco TDi front to HA475972 R/R (Carb) Heavy Duty front standard rear |
| NRC4234 | 978.84 | 170.04 | 3035.86 | GREEN - YELLOW | 411.48 | 16.2 | 7 | Range Rover RH front - Left hand drive only |
| NRC4304 | 1106.06 | 170.04 | 3035 | RED - WHITE | 450 | 17.71 | 7 | Range Rover rear heavy duty |
| NRC4305 | 1069.87 | 149.03 | 2678.7 | RED - YELLOW | 436.4 | 17.18 | 7.65 | Range Rover diesel LH front - Left hand drive only |
| NRC4306 | 855.85 | 133.04 | 2375.2 | BLUE - WHITE | 417.57 | 16.44 | 7.55 | Range Rover petrol LH front - Left hand drive only |
| NRC6388 | 681.25 | 179.87 | 3211.96 | 2 BLUE | 420 | 16.53 | 7.5 | 110 rear levelled suspension drivers |
| NRC6389 | 643.97 | 330.04 | 5893.57 | 2 RED | 407 | 16.02 | 8 | 110 rear drivers non levelled / heavy duty |
| NRC6904 | 643.97 | 330.04 | 5893.57 | RED - GREEN | 395 | 15.55 | 8 | 110 rear passengers non Ievelled / heavy duty |
| NRC7OOO | 881.26 | 179.87 | 3211.96 | GREEN - WHITE | 400 | 15.75 | 7.5 | 110 rear drivers levelled suspension |
| NRC8044 | 269.77 | 154.74 | 2763.21 | 2 WHITE | 384.3 | 15.15 | 8.9 | 110 front drivers standard |
| NRC8O4S | 171.08 | 154.74 | 2763.21 | 2 YELLOW | 368.1 | 14.49 | 8.9 | 110 front passengers standard |
| NRC8113 | 449.82 | 178.2 | 3182.1 | PINK- PURPLE | 418.36 | 16.47 | 8.75 | Range Rover EFi rear (progressive) |
| NRC9446 | 819.96 | 175.07 | 3126.25 | BLUE - GREEN | 385.7 | 15.19 | 7.75 | 90 front drivers |
| NRC9447 | 819.98 | 175.07 | 3126.25 | BLUE - YELLOW | 373.02 | 14.69 | 7.75 | 90 front passengers |
| NRC9448 | 585 | 225.09 | 4019.46 | BLUE - RED | 386.62 | 15.3 | 8 | 110 front drivers heavy duty, 130 drivers front, 90 rear drivers |
| NRC9449 | 585 | 225.09 | 4019.46 | YELLOW-WHITE | 376 | 14.8 | 8 | 110 front passenger heavy duty 130 front passenger, 90 rear passenger |
| NRC9462 | 631.97 | 225.09 | 4019.46 | GREEN - YELLOW- RED | 387.57 | 15.26 | 9 | 90 rear drivers heavy duty |
| NRC9463 | 531.97 | 127.33 | 2273.75 | GREEN - YELLOW- WHI | 376.27 | 14.81 | 9 | 90 rear passengers heavy duty |
| NTC3285 | 427.14 | 178.2 | 3182.1 | GREEN - PINK | 461.67 | 18.18 | 8.75 | Range Rover diesel rear LH - Left hand drive diesel only |
| NTC8476 | 1034.57 | 149.89 | 2676.61 | WHITE - PINK - BLUE |  |  |  | Discovery diesel LH front - Left hand drive only |

LANDROVER SPRINGS

| Part # | Vehicle(s) | Axle | Length | Rate | Coils | Colour Code |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | inches | lb/inch |  |  |
| NRC4306 | Disco V8 '90-'96 RR '87-'91 | Front left (LHD) | 16.44 | 133 | 7.55 | blue/white |
| 572315 | Disco V8 '90-'96 RR '87-'91 | Front right (LHD) Front (RHD) | 15.4 | 133 | 7.18 | blue |
| NRC2119 | Disco TDI to HA475972 RR Carb front HD | Front (RHD) Front right (LHD) | 16.13 | 150 | 7.63 | green |
| NRC 8476 | Disco Diesel LHD | Front right | 16.55 | 150 | 7 | white/blue/pink |
| NRC4305 | Disco 200TDI and RR diesel | Front left (LHD) | 17.18 | 150 | 7.65 | red/yellow |
| NRC4234 | RR LHD | Front right | 16.20 | 170 | 7.00 | green/yellow |
| NRC9446 | D90, Disco HD | Front drivers side | 15.19 | 175 | 7.75 | blue/green |
| NRC9447 | D90, Disco HD | Front passenger side | 14.80 | 175 | 7.75 | blue/yellow |
| NTC8477 | Disco w/ winch kit | Front | 18.18 | 178 | 8.75 | green/blue/yellow |
| NRC8044 | D110 | Front left | 15.13 | 155/215^ | 9^ | white/white |
| NRC8045 | D110 | Front right | 14.49 | 155/215^ | 9^ | yellow/yellow |
| NRC9448 | D110 HD, D130 | Front drivers side | 15.31 | 225 | 8 | blue/red |
| NRC9449 | D110 HD, D130 | Front passenger side | 14.80 | 225 | 8 | yellow/white |
| Rear Springs |  |  |  |  |  |  |
| NRC2119 | RR | Rear * | 16.13 | 150 | 7.63 | green |
| NRC4305 | RR | Rear * | 17.18 | 150 | 7.65 | red/yellow |
| NRC4304 | RR HD | Rear | 17.71 | 170 | 7 | red/white |
| NTC3285 | RR '87-'91 | Rear left * | 17.13 | 180 | 10.21 | green/pink |
| NRC8113 | RR '87-'91 | Rear right * | 16.47 | 180 | 8.75 | pink/purple |
| NRC6388 | D110 with load leveler | Rear passenger side* | 16.53 | 180 | 7.5 | blue/blue |
| NRC7000 | D110 with load leveler | Rear drivers side* | 15.75 | 180 | 7.5 | green/white |
| NTC5527 | Disco V8 '90-'93 | Rear (RHD) | 16.06 | 210 | 9.2 | red/pink |
| NRC8044 | Disco | Rear | 15.13 | 155/215^ | 9^ | white/white |
| NRC8045 | Disco | Rear | 14.49 | 155/215^ | 9^ | yellow/yellow |
| NRC9462 | D90 HD | Rear drivers side | 15.25 | 210/240^ | 9.2 | green/yellow/red |
| NRC9463 | D90 HD | Rear passenger side | 14.81 | 210/240^ | 9.2 | green/yellow/white |
| NRC9448 | D90 | Rear | 15.31 | 225 | 8 | blue/red |
| NRC9449 | D90 | Rear | 14.80 | 225 | 8 | yellow/white |
| RKB101111 | D110 (VIN XZ159807 up) | Rear | 16.10 | 270 to 420 | 8.8 | brown/putple |
| NRC6904** | 110 HD | Rear passenger side | 15.55 | 330 | 8 | red/green |
| NRC6389** | 110 HD | Rear driver side | 16.02 | 330 | 8 | red/red |
| RRC3266 | 130 Inner Helper Springs | Rear | ?? | 140 | 9 | ?? |

Old Man EMU Coil Springs for Land Rovers

| Part # | Vehicle(s) | Axle | Application | Length (DS) | Length (PS) | Rate | Bar | Coils |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Front |  |  |  | inches | inches | lb/inch | mm |  |
| 760 | Disco, RR (V8) | Front | Light Duty - up to 20 kg extra weight | 15.1 | 14.6 | 180-250 | 16 |  |
| 761 | All V8 | Front | Medium Duty - 20 to 50 kg extra weight | 15.7 | 15.4 | 200 | 16 | 9.1 |
| 751 | All V8 | Front | Heavy Duty - 50 to 110 kg extra weight | 15.3 | 14.9 | 230 | 16 |  |
| 767 | Al diesel | Front | Hevy Duty – 110 kg extra weight plus | 15.1 | 14.9 | ? |  |  |
| 776 | D2 | Front | Light Duty - up to 20 kg extra weight | 16.2 | 15.4 | 160 |  |  |
| 777 | D2 | Front | Medium Duty - 20 to 50 kg extra weight | 15.6 | 14.8 | 180 |  |  |
| 779 | D2 | Front | Heavy Duty - 50 to 110 kg extra weight | 16.2 | 15.2 | 180 |  |  |
| Rear |  |  |  |  |  |  |  |  |
| 752 | D110 | Rear | Light Duty | 17.3 | 17.3 | 270 | 17 |  |
| 764** | Disco, RR, D90 | Rear | Medium Duty | 16.9 | 16.5 | 240 |  | 9.5 |
| 781 | Disco, RR, D90, D2 | Rear | Medium/Heavy Duty | 16.7 | 16.3 | 290 |  |  |
| 762 | Disco, RR, D90, D2 | Rear | Heavy Duty - 100 kg constant load | 16.5 | 16.1 | 300-340 | 18 | 9.5 |
| 763 | Disco, RR, D90, D2 | Rear | Very Heavy Duty - 200 kg constant load | 16.5 | 16.1 | 360 | 18 |  |
| 755 | D110 | Rear | Heavy Duty - 300 kg constant load | 17.1 | 17.1 | 280?? | 19 |  |
| 754 | D110 | Rear | Very heavy Duty - 500 kg constant load | 17.5 | 17.1 | 420 | 21 |  |
| Old |  |  |  |  |  |  |  |  |
| 757 | All V8 | Front | Medium Duty, Replaced by 761 | 15.9 | 15.6 | 180-250 | 16 |  |
| 758 | Disco, RR, D90 | Rear | Medium Duty, Replaced by 764 | 16.9 | 16.5 | 200 | 17 |  |
| 759 | Disco, RR, D90 | Rear | Heavy Duty, Replaced by 762 | 18.1 | 17.7 | 260-300 | 17 |  |

* RR - classic RR only