1999-2004 JEEP GRAND CHEROKEE WJ 4" LIFT KIT

by

ערכת הגבהה "4 ל-Grand Cherokee WJ

- Glide-Ride Leaf Spring Suspension Systems

Shotpeening increases tensile strength and reduces fatigue while lessening the material thickness required. The thinner material allows for increased wheel travel and provides a softer ride.

Bolt-style leaf alignment clamps prevent leaves from fanning out and allow the leaves to twist and flex, yielding higher articulation.

Each spring is preset using hydraulic stress to reduce fatigue in the steel and prevent sagging.

Some applications include leaf springs with steel encased rubber bushings. Other applications include blue urethane bushings.

Anti-friction pads are placed between the leaves to reduce friction within the spring and noise from metal-to-metal contact.

Tapered and roll-pointed leaves add flexibility and provides an even deflection rate. Tapering allows a smoother transition between leaves as compression takes place, resulting in a softer ride with increased wheel travel.

- Pro-Ride Coil Spring Suspension Systems

Springs are either cold wound with chrome silicon 9254 or hot wound with 5160 to guarantee long life.

BDS coil springs are designed to withstand full coil bind. In other words, you can smash our coils to totally flat and they will return to their original height.

A polyester based powder coat is applied to ensure flexibility and durability.

Each spring is preset with a hydraulic stress to reduce fatigue in the steel and prevent sagging.

Shotpeening increases tensile strength and allows for maximum wheel travel without sacrificing the life of the spring.

- Proudly Made in the USA

All BDS Suspension products are made in America. Buying American ensures that you are getting quality products made from quality materials. Just as important is the fact that buying American keeps your hard-earned money in the American economy. This is one of the biggest ways we as consumers can help revitalize the American economy and strengthen the US dollar. It doesn’t matter whether you are buying clothing, groceries or parts for your truck…buy American and buy with confidence that your money will stay in America.

חברת BDS מציעה קיט "4 לרכבי גרנד-צ'רוקי 2004-1999 עם קפיצים ספירליים חדשים מקדימה ומאחור וזרועות רדיוס תחתונות מתכווננות כדי לאפשר כיוון פרונט נכון. הזרועות מגיעות עם פטמות גירוז.

כדי להחזיר את זוויות ההיגוי למצב הנכון, מצורפות לקיט זרוע פיטמן מונמכת והנמכה למוט פנהרד.  הקיט כולל גם הנמכה לטרנספר, מוטות מייצב אחוריים כדי לספק מהלכי מתלה ברמה הגבוהה ביותר ובנוסף, יש גם תושבת הגבהה לחיבור המשולש האחורי.

Base Kit Id 448H קיט 4" מחיר 8500 ₪ לפני מע"מ

Base Kit Id 447H קיט 2" הכולל רק 4 בולמים ו4 קפיצים- 3900 ₪ לפני מע"מ